Week 4

- Write an assembly program to find whether a given string is a palindrome or not, using stack operations

.data

string: .byte 77,79,77

len: .word 3                          # Length of string

result: .word 0

.text

.globl _start

_start:

la x5, string       # Load base address of string

lw x6, len         # Load length of string

li x7, 0           # Left index (i)

addi x6, x6, -1    # Right index (j = length - 1)

loop:

bge x7, x6, palindrome   # If left >= right, it's a palindrome

# Load characters at left and right indices

add x8, x5, x7     # Address of left char

add x9, x5, x6     # Address of right char

lb x8, 0(x8)       # Load left char

lb x9, 0(x9)       # Load right char

bne x8, x9, not_palindrome  # If chars don't match, not a palindrome

addi x7, x7, 1     # i++

addi x6, x6, -1    # j--

j loop

palindrome:

li x8, 1           # Set result to 1

j store_result

not_palindrome:

li x8, 0           # Set result to 0

store_result:

la x5, result      # Load address of result

sw x8, 0(x5)       # Store the result

  - Write an assembly program to search a given number in an array

.data

a: .byte 0x03, 0x42,0x33,0x12

b: .byte 0x03

.text

la x10,a              #address of array

la x11,b             #address of the no to searched

lb x16,0(x11)              #no to be searched

addi x20,x0,4              #no of loops

addi x27,x0,0

loop: lb x12,0(x10)        #loading the values of the array

beq x12,x16,exit     #checking if no is present in array

addi x27,x27,1       #incrementing the loop

beq x27,x20,exit2    #cheching for the loop iteration i<4

addi x10,x10,4       #incrementing the array address

beq x0,x0,loop

exit: addi x25,x0,1       #if x25 is 1 the no is present

exit2: nop