Week 5

- Write an assembly program to check whether a given number in an array of elements is divisible by 9

.data

n: .word 0x000000081, 0x00000180  # Given number to check divisibility by 9

.text

la x5, n          # Load address of the number into x5

lw x6, 0(x5)      # Load the value of n into x6

addi x7, x0, 0x0F   # Mask for the lower 4 bits (0x0F = 00001111)

addi x28, x0, 0      # Initialize sum of digits to 0

addi x29, x0, 8      # We have 8 digits to process (since 32-bit number, 8 digits of 4-bits)

loop:

and x18, x7, x6      # Isolate the lower 4 bits of x6 (digit)

add x28, x28, x18    # Add the isolated digit to the sum

srli x6, x6, 4       # Right shift x6 by 4 to process the next digit

addi x29, x29, -1    # Decrement loop counter

bne x29, x0, loop    # Repeat the loop if there are more digits to process

new:

# Now we have the sum of the digits in x28

# Check divisibility by subtracting 9 repeatedly

addi x30, x0, 9       # Load 9 into x30 for subtraction loop

check_divisibility:

blt x28, x30, done    # If sum is less than 9, exit loop

sub x28, x28, x30     # Subtract 9 from the sum

j check_divisibility  # Repeat until sum is less than 9

done:

# If the final sum is 0, it's divisible by 9

beq x28, x0, divisible

not_divisible:

li x31, 0            # Set x31 to 0 (not divisible by 9)

j end

divisible:

li x31, 1            # Set x31 to 1 (divisible by 9)

end:

# x31 contains the result: 1 if divisible by 9, 0 otherwise

nop

- Write an Assembly Program for the following C code:

main() { unsigned short int a[11] = {0x1234, 0x5678, ...}; unsigned short int b[11] = {0x1234, 0x5678, ...}; unsigned short int c[11] = {0x1234, 0x5678, ...}; for(i = 0; i < 10; i++) { c[i] = a[i] * b[i] + c[i-1]; } } ```

.data

a:  .half 0x1234, 0x5678, 0x1111, 0x2222, 0x3333, 0x4444, 0x5555, 0x6666, 0x7777, 0x8888, 0x9999  # Array a

b:  .half 0x1234, 0x5678, 0x1111, 0x2222, 0x3333, 0x4444, 0x5555, 0x6666, 0x7777, 0x8888, 0x9999  # Array b

c:  .half 0x1234, 0x5678, 0x1111, 0x2222, 0x3333, 0x4444, 0x5555, 0x6666, 0x7777, 0x8888, 0x9999  # Array c

.text

main:

la x10, a                # Load base address of array a into x10

la x11, b                # Load base address of array b into x11

la x12, c                # Load base address of array c into x12

li x14, 10               # Loop counter (i = 0 to 9)

li x13, 0                # i = 0, loop index

loop:

beq x13, x14, exit       # If i == 10, exit the loop

lh x15, 0(x10)           # Load a[i] into x15

lh x16, 0(x11)           # Load b[i] into x16

mul x17, x15, x16        # Multiply a[i] and b[i], result in x17

beq x13, x0, first_iter  # If i == 0, skip adding c[i-1]

lh x18, -2(x12)          # Load c[i-1] into x18

add x17, x17, x18        # Add c[i-1] to the result

first_iter:

sh x17, 0(x12)           # Store result into c[i]

addi x10, x10, 2         # Move to a[i+1]

addi x11, x11, 2         # Move to b[i+1]

addi x12, x12, 2         # Move to c[i+1]

addi x13, x13, 1         # Increment loop index

j loop                   # Jump back to loop

exit:

nop                      # Exit point